Infrastructure Schema v2.15

v2.15  2001/01/15

Changed the name of the SuperDevice table to Device.  Added a description of the Module table which was missing from the previous version.

v2.14  2000/09/15

Schema completely revamped.  Almost everything from v2.03 has been replaced.

v2.03  2000/08/10

Added UnitType table, and changed “DeviceTypes” table name to “DeviceType”.  (skipped revision 2.02, so that the rev number would match that of the ERD)

v2.01  2000/08/03

Made corrections and described columns in more detail so that they match the tables in the database.

v1.91 2000/07/13

Made several syntax changes and cosmetic cleanups.  Integrated a description of the example data.

v1.9 2000/07/06

Changed the way device slot connections are modeled.  Connections are now modeled from output pin on one device to output pin on the next, thus removing the need for tracking input plexors in PlexorSlot.  Connections are validated by walking the Device Chain and recovering response and device attribute information.

Removed validity time range from the DeviceBind table.  DeviceBind is only used to bind DeviceSlots to generic devices(that have default response info) and summary devices(such as the device representing the system at each site).  All ActualDevices that may be real are bound via the DeviceSubstitution table, instead of the DeviceBind table.  Only permanent binds are done via DeviceBind and thus a time range is not needed.

Changed the context module field(idDeviceSlotModule) to idDeviceModule so that it is keyed on the ActualDevice table instead of the DeviceSlot table.  This is a more flexible approach that allows details of a device module to be changed in the context of that module instead of just in the context of an entire system.

Note:  There is a known limitation to the current schema, in that it does not allow the ability to change the structure of an actual device.  So if you create an ActualDevice, then you can always have multiple response sets for that device over multiple time intervals, but that device may have only one internal structure for its entire life.  This limitation was deemed unimportant, because there were no existing or imagined devices envisioned that performed internal structural reconfigurations.

v1.7 2000/06/19

Changed the way that device connectivity was modeled.  Device plexors(inputs/outputs) were formerly a property of the ActualDevice, and were independent of the plexors of the DeviceSlots in which they were placed.  Plexors are no longer a property of the device.  All devices must have the exact plexors and internal structure described by the DeviceSlots in which they are placed.  ActualDevice no longer has any device chain properties, it only models the spectral response and identification properties(serial# and location) of a device.  This way we may model device modules that are field-configurable to have different inputs and outputs.  A SubstituteDeviceSlot table similar to SubstituteDevice has been added to allow for substituting a DeviceSlot and thus the structure of a device module within the context of another.

v1.6 Revision 2 2000/06/15

Fixed two problems in the logic for recovering the response for a channel in the  “Steps for Discovering the Response for a Channel” section.  Step 8 had logic that returned to step 6 if a device model was found, thus causing an infinite loop if  a module contained a device that was derived from another device which was in a second module that the original module was derived from.  Step 22 did not have logic to handle the case when a signal terminated in a device such in a sensor where the signal ends/begins.

v1.5  2000/06/13 Initial version

INFRASTRUCTURE  SCHEMA	3

Nomenclature	3

Device Chains	3

Device Slots	3

Plexors	3

Modules	3

ENTITY RELATIONSHIP DIAGRAM	3

Cooked Infrastructure	3

ChanT	4

Core	4

ActualDevice	4

DeviceBind	4

DeviceSlot	4

DeviceType	5

GenericDevice	5

ModuleEntry	5

ModuleTemplate	5

SlotType	6

Device	6

Physical/Inventory	6

RealDevice	7

DeviceLocation (PLEASE IGNORE FOR NOW)	7

Location (PLEASE IGNORE FOR NOW)	7

Atomic Functions	7

FunctionBind	7

PZFilter (PLEASE IGNORE FOR NOW)	8

PolesZeroes (PLEASE IGNORE FOR NOW)	8

RELATIONSHIPS	9

ActualDevice	9

ChanT	9

DeviceBind	9

DeviceSlot	9

DeviceType	10

FunctionBind	10

GenericDevice	10

Module	11

ModuleEntry	11

ModuleTemplate	11

RealDevice	12

SlotType	12

Device	12

REFERENTIAL INTEGRITY CHECKS	12

ActualDevice	12

ChanT	12

DeviceBind	12

DeviceSlot	13

DeviceType	13

FunctionBind	13

GenericDevice	13

Module	13

ModuleEntry	13

ModuleTemplate	13

RealDevice	13

SlotType	13

Device	13

TRAVERSING THE CHANNEL	14

Steps for Discovering the Response for a Channel	14

Steps	14

Writing Dataless SEED Volumes	15

DOCUMENTS	15

EXAMPLE	15

# INFRASTRUCTURE  SCHEMA

This document covers the EW5 infrastructure schema as of version 2.14.  Please see the accompanying

infra_v2.14.* file for the Entity Relationship Diagram.  Examples files are also available in Excel format depicting an example USNSN setup.

##### Versioning Note

Please note that the schema (EW5 Infrastructure Schema) was completely revamped prior to v2.14 of this document.  Version numbering is confusing, and went backwards from 2.03 to v1.10, which was derived from 1.9.  From 1.10, the version changed  to 1.11 to 1.12 (with several revs), 1.13 and finally to 1.14.  v1.14 was renamed to v2.14 to make it the highest version number of any of the EW5 infrastructure schemas, and also to make it comparable to the other existing EW5 schemas(parametric, waveform, etc.).

## Nomenclature

### Device Chains

Devices are chained together to deliver a signal from ground motion to digital waveforms that are eventually reviewed.  This chain of devices is what we must track in order to back a signal out from waveforms to ground motion.   In the EW5 infrastructure schema, we model device chains as chains of DeviceSlots.

Device chains are created by DeviceSlots together into a chain.  Once a device chain is built it may not be changed.

### Device Slots

A DeviceSlot is simply a placeholder for a device.  A DeviceSlot has a certain structure (X outputs, Y inputs), that all devices which plug into it must match.  It describes how a device interfaces with the outside world and allows someone to map how devices will be chained together without needing any actual devices.  That way different devices (of the same general type) can be swapped in and out of the device slots without having to remap the structure of the chain that links all of the slots together.  A DeviceSlot is somewhat akin to a socket in a strand of Christmas tree lights.  The sockets are all hard wired together  in the strand, but the actual light bulbs don’t need to be around for the wire strand to be created.  Later, various light bulbs of the same general type can be screwed in and out of a particular socket, without having to rewire the strand.

### Plexors

Plexors represent logical input an output pins for a device or DeviceSlot.  These days very little is done with physical pins.  Most input and output signals are multiplexed through a single physical connection.  Even worse, the outputs are software configurable, so there may be 1000 different possible outputs.  In practice, even the most complex device will have less than twenty output configurations used.  Plexors are used to chain devices together.  Because some devices have multiple plexors, it is not specific enough to say that device A is connected to device B, but instead you must say that plexor 5 of device A is connected to plexor 1 of device B.

### Modules

Modules are a type of device that has multiple inputs and outputs and contains other devices within it.  A datalogger is an example of a module.  A datalogger (potentially) manipulates multiple channels of data and there are multiple devices (digitizers, decimators, etc…) inside the datalogger.

## ENTITY RELATIONSHIP DIAGRAM

Purpose:

This section describes the entities depicted in the EW5 Infrastructure Schema Entity Relationship Diagram(ERD).  The diagram depicts the method for storing station infrastructure and inventory control data in the database.  The ERD itself is a separate document (infra_v2.14.gif or infra_v2.14.vsd).

### Cooked Infrastructure

The following tables are part of the Cooked Infrastructure schema.

#### ChanT

See Cooked Infrastructure Schema diagram and document.

### Core

The following tables are part of the Core section of the Infrastructure schema.

#### ActualDevice

Purpose:	Represent a non-generic logical device.  The set of ActualDevices is a subset of Device, and represents only those devices that are instantiated(not generic).  ActualDevice shares the Device.idDevice primary key, meaning that for every ActualDevice record, there is a matching Device record with the same idDevice. ActualDevice exists to ensure that physical devices(RealDevice) are not linked to generic devices, but only instantiated ones.

Parameters

#idDevice	Unique record identifier.  (FK to Device.idDevice)

Reviewed:	DavidK 2000/09/05

#### DeviceBind

Purpose:	Bind a device(Device) to a DeviceSlot.  A device chain is built by chaining DeviceSlots together.  However, there are no devices in a chain until devices(Devices) are bound to DeviceSlots.  DeviceBind provides the mechanism for placing a device  in a DeviceSlot for a period of time.  Time must be tracked because one device can be swapped out for another, and we must keep a historical record of the contents of the device chain.  In order to bind a device to a DeviceSlot using DeviceBind, a context is also needed.  The context is needed because chains of DeviceSlots are often generic.  Q680 dataloggers are deployed at multiple USNSN sites.  All of those Q680 dataloggers are of the same DeviceType, thus they have the same Module structure, and thus they contain the same internal device chains (chains of DeviceSlots).  Because multiple Q680 dataloggers use the same DeviceSlot for their QDP1 module, there must be some method of distinguishing between the device in the QDP1 slot at station “KRA”  and the device attached to that same DeviceSlot at station “DAV”.  The differentiation is done using the device that contains another as its context.  So the datalogger device at “DAV” would be used as the context for binding  a QDP device to the QDP1 slot for that same datalogger.  So the actual purpose of DeviceBind is to bind a device(Device) to a DeviceSlot for a period of time, with the context of another device(Device).

Parameters

*idDeviceBind	Unique record identifier.

#idContextDev	DB id of the device within which another device is being bound to a DeviceSlot. (FK to Device.idDevice)

#idDeviceSlot	DB id of the DeviceSlot to which a device is being bound.  (FK to DeviceSlot.idDeviceSlot.)

tOff	Ending time of the DeviceBind period. (ex: 9876543210.1234)

tOn	Starting time of the DeviceBind period. (ex: 9876543210.1234)

#idDevice	DB id of the device that this record binds to a DeviceSlot. .  (FK to Device.idDevice)

Reviewed:	DavidK 2000/09/05

#### DeviceSlot

Purpose:	Describe a slot in a device chain.  (See “Nomenclature” for a more complete description.) In this schema, devices are referred to as Devices.

Parameters

idDeviceSlot	Unique record identifier.

#idSlotType	DB id of a type or class of devices, such as sensor, datalogger, digitizer, decimator.  Foreign key to SlotType.idSlotType.

sSlotTypeName	Optional name of the DeviceSlot. Max length NAME_LEN characters with spaces allowed.

Reviewed:	DavidK 2000/09/05

#### DeviceType

Purpose:	Describe specific types of devices, as opposed to the more generic description given by SlotType.  Devices of the same DeviceType have the same structure both internally and externally.   This means that they will all have the same SlotType and also the same Module structure.   A device type is also given a name, such as Q680 Datalogger.  This allows a device type to discriminate on more than just SlotType and Module structure, so that someone cannot replace a Q680 Digitizer with a Q730 digitizer.

Parameters

idDeviceType	Unique record identifier.

*sDeviceTypeName

Name of a device type, such as “Q680 High Gain decimator”.  Max length NAME_LEN characters with spaces allowed.

#idModule	DB ID of the Module structure that belongs to this DeviceType.  (FK to Module.idModule.)

Reviewed:	DavidK 2000/09/05

#### GenericDevice

Purpose:	Represent a generic logical device.  The set of GenericDevices is a subset of Device, and represents only those devices that are generic (not instantiated).  GenericDevice shares the Device.idDevice primary key, meaning that for every GenericDevice record, there is a matching Device record with the same idDevice. GenericDevice exists to ensure that devices are not derived from instantiated devices, but only from generic devices.  ActualDevice and GenericDevice are mutually exclusive in that a particular device can be either an ActualDevice or a GenericDevice but not both.

Parameters

#idDevice	Unique record identifier.  (FK to Device.idDevice)

bIsRealizable	Flag indicating whether the device may be real.  Some devices are strictly logical and/or too small to be individually tracked. (TRUE=1 FALSE=0).  bIsRealizable is meant as a hint for people creating new instantiated devices.  (A new Q730 is received by a bunch of giddy seismologists/techs, and they wish to enter it into the DB system.  The bIsRealizable flag is a hint to let them know for which parts (internal devices) they should enter Serial#’s, and other data.)

Reviewed:	DavidK 2000/09/05

#### Module

Purpose:	Describea specific module structure.  A Module describes the internal structure of a device.  It inherits X inputs and X outputs from its corresponding SlotType, and then it describes how the outputs are connected to the internal DeviceSlots of the device.

Parameters

idModule	Unique record identifier.

*sModuleName

Name of a Module structure, such as “Q680 High Gain decimator structure”.  Max length NAME_LEN characters with spaces allowed.

#idSlotType	DB ID of the SlotType that belongs to this Module.  (FK to SlotType.idSlotType.)

Reviewed:	DavidK 2000/01/15

#### ModuleEntry

Purpose:	Describe an output point of a device chain within a Module.  (See “Module” under “Nomenclature” for a description.)  A module is linked to the DeviceSlots (which hold devices) within it, by hooking ModuleEntries to the end of the internal device chains. (Module entries are hooked to the Output end of the device chains).

Parameters

*idModuleEntry	Unique record identifier.

#idModule	Link to the Module to which this ModuleEntry plexor belongs.  (FK to Module.idModule.)

iPlexor	The numbered plexor output. Outputs start at 1 and count up to a max of 999.

#(#idSubDevSlot	DB id of the first DeviceSlot  in the device chain for this ModuleTemplate, inside the current module.  (FK to DeviceSlot.idDeviceSlot.)

iSubPlexor)	The numbered plexor output of idSubDevSlot to which this ModuleEntry is linked (1-999).

Reviewed:	DavidK 2000/09/05

#### ModuleTemplate

Purpose:	ModuleTemplate describes the plexors(output only) of a DeviceSlot.  A device chain can be created by chaining DeviceSlots together via connecting ModuleTemplate plexors.  Two plexors are linked together using the idNextDevSlot/iNextPlexor values.  The idNextDevSlot/iNextPlexor of the first ModuleTemplate record are set to the values of idDeviceSlot/iPlexor of the second ModuleTemplate record, thus connecting an input pin of one DeviceSlot to an output pin of another.   Input plexors are not modeled in order to reduce complexity.  The output pin of device X contains the information regarding which output pin of which device it’s respective input pin is connected to.  This is somewhat redundant as multiple output pins on device X may be internally connected to a single input pin on device X.  Anomalies are checked by traversing the device tree, and gathering device info in order to build the channel info.

Parameters

*idModuleTemplate

Unique record identifier.

#idDeviceSlot	Link to the DeviceSlot to which this ModuleTemplate plexor belongs.  (FK to DeviceSlot.idDeviceSlot.)

iPlexor	The numbered plexor output. Outputs start at 1 and count up to a max of 999.

#(#idNextDevSlot

DB id of the next DeviceSlot  in the device chain for this ModuleTemplate.  (FK to DeviceSlot.idDeviceSlot.)

iNextPlexor)	The numbered plexor output of idNextDevSlot to which this ModuleTemplate is linked (1-999).

Reviewed:	DavidK 2000/09/05

#### SlotType

Purpose:	Describe types of DeviceSlots/devices.  All devices of a certain type have the same input and output units, such as digitizers that input Volts and output Counts.  All DeviceSlots (and thus the devices that can plug into them) also have the same number of inputs/outputs. A device slot  type is also given a name, such as digitizer or  12/6 datalogger, this allows a device slot to discriminate on input/output units, number of inputs/outputs and also name. Slot Types allow the schema to categorize devices by type, thus preventing someone from putting a square peg into a round hole (a digitizer ActualDevice into a datalogger DeviceSlot.  SlotTypes are differentiated from DeviceTypes in that they are more generic and only contain properties that are attributable to DeviceSlots as well as devices.

Parameters

idSlotType	Unique record identifier.

*sSlotTypeName

Name of a device slot type, such as “3-component-decimator”.  Max length NAME_LEN characters with spaces allowed.

#iNumInputs	Number of input plexors to the device/DeviceSlot. (1-999)

#iNumOutputs	Number of output plexors from the device/DeviceSlot. (1-999)

Reviewed:	DavidK 2000/09/05

#### Device

Purpose:	Represent a logical device.  Device is the core device entity.  It represents a logical device that may be physical(see RealDevice).  A Device can be a module that contains other devices, or it can be a simple device.  A Device can have functionality associated with it via FunctionBind.  A Device can be derived from some other generic device (GenericDevice) by using the idDeviceModel field as a foreign key to the idDevice field of a GenericDevice record.  Ultimately, to bee useful, a device must be placed in a device chain.  This is done by binding it to a DeviceSlot(see DeviceBind).

Parameters

idDevice	Unique record identifier.

#idModule	Link to the Module that represents the internal structure of this device. (FK to Module.idModule).  Note this field is not normalized.  idModule is rightfully a property of DeviceType, to which Device is linked.  This field has been added to Device in order to improve retrieval performance, by circumventing the DeviceType field.  idModule has been included in this table solely for performance reasons.

#idDeviceModel	Link to the GenericDevice that this Device is derived from.  (FK to GenericDevice.idDevice.)

sDeviceName	Name of this device.  Max length of NAME_LEN characters, with spaces allowed.

#idDeviceType	Link to the type of device that this device is. (Q680 Datalogger, STS-2 sensor, Q730 digitizer).  (FK to DeviceType.idDeviceType)

bIsGeneric	Flag indicating whether the device is generic.  (TRUE=1 FALSE=0).  (This serves as a shortcut to doing a join to the GenericDevice table.  (It serves only as a performance enhancement, and may be removed.)

Reviewed:	DavidK 2000/09/05

### Physical/Inventory

The following tables are part of the Physical/Inventory section of the Infrastructure schema.

#### RealDevice

Purpose:	Represent a physical device that has a serial number and possibly a location.  RealDevice associates physical properties with an ActualDevice, thus making the logical ActualDevice also physical.  Only ActualDevice records that are associated with RealDevice records represent physical devices.  Physical devices have special attributes that exclusively logical devices do not have, such as a serial number, a manufacturer, and a location.

Parameters

#idActualDevice	The DB id of the ActualDevice record that this physical device is associated with.  Foreign Key to ActualDevice.idDevice.

sSerialNum	The serial number of this physical device.  Max length of 20 characters, with spaces allowed.

#idManufacturer	The DB id of the Manufacturer of this device.  FK to Manufacturer.idManufacturer.

Reviewed:	DavidK 2000/07/20

#### DeviceLocation (PLEASE IGNORE FOR NOW)

Purpose:	Associate a RealDevice with a Location for a period of time.

Parameters

#idActualDevice	The DB id of the ActualDevice record that this physical device is associated with.  FK to ActualDevice.idActualDevice.

tOff	Ending time of the DeviceLocation period. (ex: 9876543210.1234)

tOn	Starting time of the DeviceLocation period. (ex: 9876543210.1234)

#idLocation	The DB id of the Location to which the ActualDevice is bound for a given time period.  FK to Location.idLocation.

Reviewed:	DavidK 2000/07/20

#### Location (PLEASE IGNORE FOR NOW)

Purpose:	Represent a physical location.

Parameters

idLocation	Unique record identifier

dLat	Latitude of the location (degrees 321.123)

dLon	Longitude of the location (degrees 321.123)

dElev	Elevation above sea level of the location (meters 54321.1)

sLocationName	Name of the location.  Maximum 80 characters, with spaces allowed.

sLocationDescription

Description of the location.  Maximum 255 characters, with spaces allowed.

Reviewed:	DavidK 2000/07/20

### Atomic Functions

The following tables are part of the Atomic (atomic functions) section of the Infrastructure schema.

#### FunctionBind

Purpose:	Bind an atomic function to a device(Device) for a time period.   This is how functionality and response information is tied to a device.  Each device performs potentially multiple modifications to the data signal as the signal passes through it.  Each of these modification is recorded as a function, such as PZ Filter, FIR Filter, unit change, gain, etc…

Parameters

*idFunctionBind	Unique record identifier

#idDevice	The DB id of the Device record that a function is being bound to

tiFunction	Table identifier of the function type that is being bound to the Device

idFunction	DB id of the function record of type tiFunction that is being bound to the Device

tOff	Ending time of the FunctionBind period.

tOn	Starting time of the FunctionBind period.

bOverridable	Flag indicating whether this device property should be overridden by a device when the device is instantiated from a generic device to which this property belongs.  Device properties such as gain are usually device dependent, so that each CMG3-N device has a different gain.  FIR Filter properties of decimators are usually the same for each instance of the same model of decimator, and thus are not device dependent.  (TRUE=1 FALSE=0).

#### PZFilter (PLEASE IGNORE FOR NOW)

Purpose:	Provide a Poles & Zeroes atomic function.

Parameters

idFunction	Unique record identifier

sFiltername	A name assigned to the filter by its creator. Maximum 50 characters, with spaces allowed.

Reviewed:	DavidK 2000/08/03

#### PolesZeroes (PLEASE IGNORE FOR NOW)

Purpose:	Represent a pole or zero in a Poles and Zeroes function.

Parameters

#idFunction	The DB id of the PZFilter function of which this record is a part.

iPoleOrZero	Flag indicating whether this record is a Pole (1) or a Zero (0)

dReal	Real component of this PoleZero complex number. (7654321.1234567)

dImaginary	Imaginary component of this PoleZero complex number. (7654321.1234567)

Reviewed:	DavidK 2000/08/03

## RELATIONSHIPS

#### ActualDevice

RealDevice

An ActualDevice is a logical device that may be real, meaning that it may have a Manufacturer and Serial#.  All ActualDevices that are real, are associated with a RealDevice that holds the Manufacturer, Serial#, and other external identification information.  The relationship is 1-to-1, and there must exist an ActualDevice for each RealDevice.

Device

Device is the superclass for all devices.  ActualDevice is a subclass of Device, and thus every ActualDevice has one associated Device record.  The relationship is 1-to-1, and there must exist a Device for each ActualDevice.

#### ChanT

DeviceSlot

ChanT represents a data channel for a given time period.  That data channel is composed of a string of devices.  ChanT is linked to an output plexor(ModuleTemplate) from the final DeviceSlot(output side) in the chain, identified by idDeviceSlot/iPlexor.  The idDeviceSlot identifies the DeviceSlot that is last in the chain.  Many ChanT records may be linked to the same DeviceSlot, but only one ChanT record should be linked to one output plexor (ModuleTemplate) for a given time period.  The relationship is optional(a DeviceSlot can exist without a ChanT and vice versa) and is many-to-1(ChanT-to-DeviceSlot).

ModuleTemplate

ChanT represents a data channel for a given time period.  That data channel is composed of a string of devices.  ChanT is linked to an output plexor(ModuleTemplate) from the final DeviceSlot(output side) in the chain, identified by idDeviceSlot/iPlexor.  Multiple ChanT records can be linked to the same output plexor(ModuleTemplate), but only one ChanT record should be linked to one ModuleTemplate for a given time period.  The relationship is optional(a ModuleTemplate can exist without a ChanT and vice versa) and is many-to-1(ChanT-to-ModuleTemplate).  However, if time is kept constant, then the relationship is 1-to-1.

#### DeviceBind

DeviceSlot

DeviceBind binds a device(Device) to a DeviceSlot for a given time period and context.  More than one device can be bound to a DeviceSlot over time, but within a given context, only one device may be bound to a DeviceSlot at any given time.  The relationship is many-to-1(DeviceBind-to-DeviceSlot), but is 1-to-1 if time and device context are held constant.  A DeviceSlot must exist for each DeviceBind, since you cannot bind a device to a slot if you do not have a slot.

Device (2)

There are two relationships between Device and DeviceBind.  A DeviceBind record binds a device(Device) to a DeviceSlot within the context of another device.  The first relationship (DeviceBind.idContextDevice -> Device.idDevice) identifies the device context within which the bind takes place.  The second relationship (DeviceBind.idDevice -> Device.idDevice) identifies the device that is bound to the DeviceSlot within the idContextDevice context.  Both relationships are 1-to-many(Device-to-DeviceBind), and a DeviceBind requires both a context device(idContextDevice) and a device(idDevice) to bind to a slot, in order to exist.

#### DeviceSlot

ChanT

[See ChanT->DeviceSlot relationship description]

DeviceBind

[See DeviceBind->DeviceSlot relationship description]

ModuleEntry

A ModuleEntry links outputs from a module to device chains within that module, by connecting a Module plexor to a ModuleTemplate (output plexor) of the last device in an internal chain.  The ModuleTemplate is identified by idSubDevSlot/iSubPlexor.  iSubDevSlot identifies the DeviceSlot of the ModuleTemplate.

ModuleTemplate(2)

Each ModuleTemplate record is potentially associated with two DeviceSlots.  A ModuleTemplate identifies an output plexor of a DeviceSlot, thus ModuleTemplate -> idDeviceSlot identifies the DeviceSlot to which the ModuleTemplate belongs.  The relationship is many-to-1 (ModuleTemplate-to-DeviceSlot) as a DeviceSlot may have more than one output plexor.  A DeviceSlot must exist for each ModuleTemplate, as you cannot have output pins on a slot if you have no slot.

A ModuleTemplate record may also link the current ModuleTemplate to its corresponding next ModuleTemplate in a device chain.  A ModuleTemplate then is linked to another ModuleTemplate, which belongs to a DeviceSlot.  ModuleTemplate -> idNextDevSlot identifies the DeviceSlot to which the next ModuleTemplate in the chain belongs.  The relationship is many-to-1 (ModuleTemplate-to-DeviceSlot) as more than one ModuleTemplate can be linked to ModuleTemplates that belong to the same DeviceSlot.  idNextDevSlot is not required for a ModuleTemplate to exist, as the current ModuleTemplate may be the end(beginning) of the chain, and thus there would not be an idNextDevSlot.

SlotType

DeviceSlots and devices that plug into them have certain common attributes, including the number of inputs and outputs.  DeviceSlots and devices for which these common attributes have the same values are said to have the same SlotType.  Multiple DeviceSlots can have the same SlotType, and each DeviceSlot can have only a single SlotType.  The relationship is 1-to-many (SlotType to DeviceSlot), and for each DeviceSlot there must be a single SlotType.

#### DeviceType

Module

A DeviceType is a generic class of device that has a particular name and structure, such as “Quanterra Q680 Datalogger”.  Module represents the internal structure of a device, and all devices of a particular DeviceType have the same structure.  The relationship is 1-to-many (Module to DeviceType), in practice the relationship should be very close to 1-to-1, as multiple DeviceTypes will not usually share the same structure.  The structure for each DeviceType must be specified, so each DeviceType requires a Module structure.

Device

Devices(Device) are classified into DeviceTypes.  Multiple devices can have the same DeviceType, and in order to derive a Device from a GenericDevice, both devices must have the same DeviceType.  The relationship is 1-to-many (DeviceType to Device), and each Device must have a single DeviceType.

#### FunctionBind

AtomicFunctions

FunctionBind binds a single Device to a single atomic function.   An atomic function may be bound to more than one Device, but each FunctionBind binds a single atomic function to a single Device.  The relationship is 1-to-many (atomic function to FunctionBind) since each FunctionBind record binds only a single atomic function, yet an atomic function can be bound by multiple FunctionBind records (to multiple Devices or to the same device over multiple time periods).  A FunctionBind record cannot exist without an atomic function, since it is hard to bind a function to a device when you do not have a function.

Device

FunctionBind binds a single device(Device) to a single atomic function.   A device may be bound to more than one atomic function, but each FunctionBind binds a single atomic function to a single Device.  The relationship is 1-to-many (Device to FunctionBind) since each FunctionBind record binds only a single device, yet a device can be bound by multiple FunctionBind records (to multiple atomic functions or to the same atomic function over multiple time periods).  A FunctionBind record cannot exist without a device, since it is hard to bind a function to a device when you do not have a device.

#### GenericDevice

Device(2)

A GenericDevice is a logical device that may not be real, meaning that it will never have a Manufacturer and Serial#. Device is the superclass for all devices.  GenericDevice relates to Device in two ways.  First, GenericDevice is a subclass of Device, and thus every GenericDevice has one associated Device record.  The relationship is 1-to-1, and there must exist a Device for each GenericDevice.  The second method by which GenericDevice and Device are related, is that a Device can be derived from a GenericDevice by setting Device -> idDeviceModel to the value of GenericDevice -> idDevice.  A Device can be derived from a single GenericDevice, so that it may inherit the properties and structure of the GenericDevice as defaults.  The relationship is 1-to-many (GenericDevice to Device) since more than one Device can be derived from the same GenericDevice.  The relationship is optional, meaning that neither a GenericDevice nor a Device is dependent upon the other.

#### Module

DeviceType

[See DeviceType ->Module relationship description]

ModuleEntry

Module represents the overall internal structure of a device.  Each ModuleEntry represents the internal device chain structure for a single output plexor(ModuleTemplate).  A module should have exactly one ModuleEntry for each ModuleTemplate in the DeviceSlot to which it is bound.  This should be ensured by DeviceSlot->idSlotType = Module->idSlotType, however there is no method in the current schema for ensuring that SlotType->iNumOutputs for idSlotType = Module->idSlotType is equal to the number of records in ModuleEntry with idModule = Module->idModule.  The relationship is 1-to-many (Module to ModuleEntry), and a corresponding Module is required for a ModuleEntry to exist.

Device

Every device(Device) has an internal structure that is represented by Module.  Even simple devices that have only one input and one output, will have a Module structure (that is NULL or empty).  Multiple devices may share the same Module structure, so the relationship is 1-to-many (Module to Device), and a Device cannot exist without a Module structure.

#### ModuleEntry

DeviceSlot

[See DeviceSlot ->ModuleEntry relationship description]

Module

[See Module->ModuleEntry relationship description]

ModuleTemplate

ModuleEntry connects an output plexor of a device to a device chain internal to that device.    ModuleEntry->idSubDevSlot/iSubPlexor points at the output plexor(ModuleTemplate) of the last DeviceSlot in the internal device chain.  A ModuleEntry can be connected to only one ModuleTemplate, but potentially multiple ModuleEntry’s can be connected to the same ModuleTemplate.  The relationship is 1-to-many (ModuleTemplate to ModuleEntry) and neither entity requires the other.  Example:  If you have a Q680 datalogger with 12 outputs, and it was internally connected to a high-gain decimator DeviceSlot, then you would have a ModuleEntry for each of the high-gain outputs of the datalogger, and each ModuleEntry would be connected to a ModuleTemplate for the outputs of the high-gain decimator slot.

#### ModuleTemplate

ChanT

[See ChanT->ModuleTemplate relationship description]

DeviceSlot(2)

[See DeviceSlot->ModuleTemplate relationship description]

ModuleEntry

[See ModuleEntry->ModuleTemplate relationship description]

ModuleTemplate

DeviceSlots are chained together into device chains by connecting ModuleTemplates of DeviceSlots.  The ModuleTemplate represents an output pin of a DeviceSlot.  It can be linked to the output pin of the next DeviceSlot in the chain.  (In the REAL world, wiring connects an output pin of a device to an input pin of the same device.  That input pin can then be connected to the output pin of another device, thus chaining the devices together.  In this schema we have not modeled input pins in order to reduce complexity and improve performance.  So if you have a DeviceSlot that has 6 outputs but only 1 input, and you connect that DeviceSlot to another, then you must model that connection for each of the 6 outputs instead of for just the one input.  So in the current schema, we model the output pin of one DeviceSlot being connected to an output pin of the next DeviceSlot and thus bypass input pins.  Note:  In most of the documentation, the term pin has been replaced with the term plexor, in order to avoid any bad stigmatism that may go along with the expression “pin number”  ).  A ModuleTemplate(on the output side of the chain) can be chained to at most one other ModuleTemplate (on the input side), however multiple output side ModuleTemplates can be connected to the same input side ModuleTemplate.  (In this context, “output side” and “input side” describe sides of the device chain relative to signal travel.  Given two DeviceSlots chained together, the one closest to the seismometer is said to be on the “input side” and the one closest to the final waveform data is on the “output side”.  The relationship is 1-to-many (input side ModuleTemplate to output side ModuleTemplate), and one ModuleTemplate does not require another.

#### RealDevice

ActualDevice

[See ActualDevice->RealDevice relationship description]

#### SlotType

DeviceSlot

[See DeviceSlot->SlotType relationship description]

Module

[See Module->SlotType relationship description]

#### Device

ActualDevice

[See ActualDevice->Device relationship description]

DeviceType

[See DeviceType-> Device relationship description]

FunctionBind

[See FunctionBind-> Device relationship description]

GenericDevice(2)

[See GenericDevice-> Device relationship description]

Module

[See Module-> Device relationship description]

## REFERENTIAL INTEGRITY CHECKS

Below is a list of checks that must be done prior to or as a part of data insertion for each table.

#### ActualDevice

- Device must contain a matching idDevice for ActualDevice->idDevice.

#### ChanT

- If idDeviceSlot is NOT NULL, then a ModuleTemplate record must exist, containing a matching idDeviceSlot and iPlexor.  A DeviceSlot record with matching idDeviceSlot must also exist, but that is implicit in the ModuleTemplate requirement.

#### DeviceBind

- A DeviceSlot record must exist with a matching idDeviceSlot.

- A Device record must exist with Device.idDevice matching DeviceBind.idContextDev.

- A Device record must exist with Device.idDevice matching DeviceBind.idDevice.

#### DeviceSlot

- A SlotType record must exist with a matching idSlotType.

#### DeviceType

- A Module record must exist with a matching idModule.

#### FunctionBind

- A Device record must exist where Device.idDevice = FunctionBind.idDevice.  (Note: Ray had talked about only allowing functionality to be bound to simple devices.  If that was done, then function bind would be restricted to ActualDevices and an ActualDevice record would be required where ActualDevice.idDevice = FunctionBind.idDevice.

- An atomic function table identified by FunctionBind.tiFunction must contain a row with “FunctionTable”.idFunction = FunctionBind.idFunction.

#### GenericDevice

- A Device record must exist with Device.idDevice matching GenericDevice.idDevice.

#### Module

- A SlotType record must exist with a matching idSlotType.

#### ModuleEntry

- A Module record must exist with a matching idModule.

- If idSubDevSlot is NOT NULL, then a ModuleTemplate record must exist with ModuleTemplate.idDeviceSlot = ModuleEntry.idSubDevSlot and ModuleTemplate.iPlexor = ModuleTemplate.iSubPlexor. A DeviceSlot record with DeviceSlot.idDeviceSlot = ModuleEntry.idSubDevSlot must also exist, but that is implicit in the ModuleTemplate requirement.

- ModuleEntry.iPlexor should not be greater than SlotType.iNumOutputs where ModuleEntry.idModule = Module.idModule and Module.idSlotType = SlotType.idSlotType.  (You should not be able to link to more output plexors than are contained in the slot.  This may not be important enough to worry about.)

#### ModuleTemplate

- A DeviceSlot record must exist with a matching idDeviceSlot.

- If idNextDevSlot is NOT NULL, then a ModuleTemplate record must exist where ModuleTemplate_A.idNextDevSlot = ModuleTemplate_B.idDeviceSlot and ModuleTemplate_A.iNextPlexor = ModuleTemplate_B.iPlexor.  A DeviceSlot record must also exist where DeviceSlot.idDeviceSlot = ModuleTemplate.idNextDevSlot, but that is implicit in the ModuleTemplate requirement.

#### RealDevice

- An ActualDevice record must exist with ActualDevice.idDevice = RealDevice.idActualDevice.

- A Manufacturer record(not depicted in diagram) must exist with a matching idManufacturer.

#### SlotType

(NONE for SlotType)

#### Device

- If idDeviceModel is NOT NULL, then a GenericDevice record must exist with GenericDevice.idDevice = Device.idDeviceModel.

- A DeviceType record must exist with a matching idDeviceType.  A Module record with a matching idModule must also exist, but that is implicit in the DeviceType requirement.

## TRAVERSING THE CHANNEL

### Steps for Discovering the Response for a Channel

The following set of steps describe the logic for walking through all of the devices in a channel and obtaining an overall spectral response for the channel.   The tables and table fields are described in the

“ENTITY RELATIONSHIP DIAGRAM” section above.  If you get confused, try following an example channel in  the .xls example file.

Note, at the start, the “Module Context Stack” is empty, the “Current Device Stack” is empty, and the main stack is empty.  FunctionBindsByPreviousDevicesInChan is empty and FunctionBindsByCurrentDevice is empty.

#### Steps

- ChanT to ModuleTemplate

ChanT with idChan and Time.  Get idDeviceSlot and iPlexor.

- Get Device for idDeviceSlot

DeviceBind using idDeviceSlot, time, and “Module Context Stack” from the bottom up.  Get idDevice.  If idDevice is NULL, then we are hopefully lost.  Abort with ERROR!

- Check for Device Models

Device using idDevice.  Get idDeviceModel.

- If idDeviceModel is NULL, then continue to the next step.  If NOT NULL, then push idDevice onto the “Current Device Stack”.  Set idDevice = idDeviceModel.  Repeat step 3.

- Get device internal structure for current channel

ModuleEntry using the current iPlexor and Device->idModule from the current idDevice.  Get idSubDevSlot and iSubPlexor.

- Check for simple device

If idSubDevSlot is NULL then skip to step 8 (Check for Atomic Functions).

- Enter the current module

idSubDevSlot is not NULL from Step 6(Check for simple device). Push idDevice onto the “Current Device Stack”.  Push the “Module Context Stack” onto the main stack.  Copy the “Current Device Stack” to the “Module Context Stack”.  Push idDeviceSlot and iPlexor onto the main stack.  Push FunctionBindsByPreviousDevicesInChan onto the main stack.  Set idDeviceSlot = idSubDevSlot and iPlexor = iSubPlexor.  Set idDevice = NULL.  Return to step 2(Get Device for idDeviceSlot).

- Check for Atomic Functions

For idDevice and each entry on the “Current Device Stack”, go to FunctionBind and retrieve each Atomic Function associated with the current devices.  Add the functions to FunctionBindsByCurrentDevice.  Traverse the “Current Device Stack” from top to bottom (generic to specific), overriding generic values with specific ones.

- Get Next DeviceSlot

ModuleTemplate using idDeviceSlot/iPlexor.  Get idNextDevSlot and iNextPlexor.

- DeviceSlot check

If idNextDevSlot is NULL, then skip to step 12(Prepare to Pop Out of Module).

- Goto Next DeviceSlot

Set idDeviceSlot = idNextDevSlot and iPlexor = iNextPlexor.  Append FunctionBindsByCurrentDevice to FunctionBindsByPreviousDevicesInChan.  Set FunctionBindsByCurrentDevice to NULL.  Goto step 2 (Get Device for idDeviceSlot).

- Prepare to Pop Out of Module

Append FunctionBindsByCurrentDevice to FunctionBindsByPreviousDevicesInChan.  Set FunctionBindsByCurrentDevice to NULL.  Move the contents of FunctionBindsByPreviousDevicesInChan to FunctionBindsByCurrentDevice.  Pop FunctionBindsByPreviousDevicesInChan off of the main stack.  Move the contents of “Module Context Stack” to the “Current Device Stack”.  Pop idDeviceSlot, iPlexor, and “Module Context Stack” from the main stack.  Pop idDevice off of the “Current Device Stack”.  If idDevice is NOT NULL then return to step 8.

- The End

Reached the end of the Channel. FunctionBindsByCurrentDevice contains the response information for the channel.

<end of steps>

### Writing Dataless SEED Volumes

To get a chain of devices for writing dataless SEED volumes with response information for each device in the chain, you would walk the same path as described above, but you would need to save the sDeviceName, idDeviceType(device type info), and FunctionBinds for each ActualDevice in the chain.  You might not use all of the FunctionBinds(write them into the SEED volume) in the chain, because some may be overridden by others in modules above them.

## DOCUMENTS

There are currently 4 documents that make up the documentation for the infrastructure schema:

- The Entity Relationship Diagram v2.14 in Visio 5 or GIF format.  (infra_v2.14.vsd | infra_v2.14.gif)

- The description document.  It describes the entities in the ERD as well as the logic required to traverse the channel to obtain response info.  (This document) (p3s2_infra_schema_description_v2.14.doc)

- The example data document.   It is an Excel file that contains the core table records required to implement two stations (DAV and KRA). (infra_example_v2.14.xls)

## EXAMPLE

The data in the MS Excel example file depicts the following:

- A generic USNSN Quanterra Datalogger Station.  This is not an actual station, but a model of the standard USNSN station setup using a Quanterra 680, a set of 3 CMG-3NSN/FBE sensors for high-gain recording, and a 3-component CMG-5 accelerometer for low-gain recording.

- A Station (DUG,US), that was placed in service on T1.  The station utilizes a standard USNSN Quanterra Datalogger Station setup, and is therefore modeled after the generic station mentioned in #1.  (Example furnished by Buland).