K2 PROGRAMMING INSTRUCTIONS

January 18, 2002

The following instructions describe how to program a K2, using Monitor Mode.  Most, but not all, K2 parameters can also be set using the QuickTalk program.

First, connect to the K2 using the QuickTalk Terminal Window, or by using the Windows Hyperterm program.  The baud rate at which the QuickTalk or Hyperterm program talks must match the baud rate setting of the K2.  Typically, K2's are set up to run at 19,200 baud.  If 19,200 baud doesn't work, try 9600 or 38,400 baud.  After connecting to the K2, type \\\<cr> to put the K2 into monitor mode.  Reset the K2 baud rate to 19,200 by typing at the monitor-mode prompt ("*"):

* BAUdrate 19200                 # Only the capitalized letters need to be typed

and then, reset the baud rate of the QuickTalk or Hyperterm program to 19,200.  Now, enter the other K2 parameters:

* AUTobaud 1                     # Disable the autobaud feature

* AQ OFF                         # Turn off data acquisition

* EDIT                           # Put the K2 into edit mode.

* AQ AUTo_delete 1               # Enable automatic event file deletion

EDIT> DEFault FACTORY            # Set all parameters to factory default values.

EDIT> STNid "JSFB"               # Station name (JSFB, for example)

EDIT> COMment "Stanford"         # Station description

EDIT> NCHannels <3 or 4>         # If using an external L4, set to 4; otherwise 3.

EDIT> GPS 60 60 0                # Synch K2 clock to GPS every 60 minutes

EDIT> UTCoffset 0                # Set the clock to GMT

EDIT> LOCation <la> <lo> <el>    # Latitude in degrees North

# Longitude in degrees East

# Elevation in meters

# Specify <la>, <lo> to 5 decimal places

EDIT> CHAnnel SEN 4 35 <sn>      # Set sensor type of channel 4 to 35.

# 35 means "Mark Products L4"

# <sn> is the serial number of the L4

EDIT> CHAnnel LOCation 1 0 0 0 0 90       # Sensor position and orientation

EDIT> CHAnnel LOCation 2 0 0 0 0 0        # <n> = meters sensor is North of K2

EDIT> CHAnnel LOCation 3 0 0 0 90 0       # <e> = meters sensor is East of K2

EDIT> CHAnnel LOCation 4 <n><e><u> 90 0   # <u> = meters sensor is above K2

EDIT> CHAnnel ID 1 HNE                    # Component name of channel 1

EDIT> CHAnnel ID 2 HNN

EDIT> CHAnnel ID 3 HNZ

EDIT> CHAnnel ID 4 EHZ

EDIT> STReam VOTers c1 1 c2 1 c3 1 c4 0   # Don't trigger on channel 4

EDIT> CHAnnel THReshold 1 0.25 0.25       # Trigger threshold in percent G

EDIT> CHAnnel THReshold 2 0.25 0.25

EDIT> CHAnnel THReshold 3 0.5 0.5

EDIT> STReam THReshold 1 1       # 1 vote to trigger; 0 votes to detrigger

EDIT> STReam FILter 1            # Apply causal, anti-alias filter to data

EDIT> STReam PREevent 10         # Number of pre-event seconds to save

EDIT> STReam POStevent 80        # Number of post-event seconds to save

EDIT> STReam MINruntime 90       # Minimum event length, in seconds

EDIT> STReam SPS 100             # Stream sample rate

EDIT> SDStreams TIMeout 0        # Use "Mode 1" = Continuous streaming

EDIT> SDStreams FORmat 1         # Compress the serial data stream

EDIT> SDStreams SAMple_rate 100  # Serial data stream sample rate

EDIT> SDStreams BUFfer_size 256  # Size of the #SSTRBUF.BIN file,

#   in 16,384 byte units

EDIT> QUIT                       # Leave edit mode and save parameters to K2

* DG

DG> GPS AVE 1                    # Enable GPS averaging

DG> Q                            # Leave diagnostic mode

* AQ ON                          # Start data acquisition