Description of Earthworm DB Schema Implementation

v2.17: 2001/05/30

Changs from v2.15

Parametric Schema

Changed name and id of CodaDur to CodaDur so that it matches CodaAmp and PeakAmp.

Re-merged OriginError information back into Origin table.  (Done due to time constraints, as adequate time was not available to test and debug code required to separate the tables as described in v2.15.

Cleaned up diagrams, standardized on version # 2.17.

v2.16 : 2001/05/16

No Description available.

v2.15: 2001/04/11

Changes from v2.14

#### Parametric Schema

Added OriginError table.

Moved Origin error information from the Origin table to the OriginError table.

Removed all references to tiExternal and xidExternal.

Added MagType table, which contains information for different magnitude types.

Added idSource to Pick table, so that Picks may now be attributed to an author.

Changed name and id of AmpCoda to CodaAmp so that it matches PeakAmp.

Replaced AmpBody table with PeakAmp table, since it handles body and surface waves.h

Replaced all references to tiMagType and sMagType with references to MagType.iMagType.

#### Cooked Infrastructure Schema

Changed idDevice to idDeviceSlot/iPlexor in ChanT table, to match the current Raw Infrastructure Schema.

#### All Schemas

Changed all references of PhaseIII to “EWDB” or similar, in order to reflect the movement of the DBMS project into Earthworm.

v2.14:  2000/05/07

Original “updated” version.

PARAMETRIC SCHEMA DIAGRAM	5

Purpose	5

Event Group	5

Event	5

Bind Group	5

Bind	5

Summary Group	6

Origin	6

MechFM	7

Magnitude	7

Link Group	7

OriginPick	7

MechFMPick	8

MagLink	8

MagType	8

Datum Summary Group	9

CodaDur	9

Ray	9

Datum Group	9

Pick	9

TCoda	10

CodaAmp	10

PeakAmp	10

EXTERNAL SCHEMA DIAGRAM	11

Purpose	11

Event Group	11

Event	11

Bind Group	11

Prefer	11

Summary Group	11

Origin	11

MechFM	11

Magnitude	11

Datum Group	11

Pick	11

TCoda	11

CodaAmp	11

PeakAmp	11

External Group	12

External Table:  DEPRECATED!!!!	12

Others	12

Source	12

ExternalEvent	12

STRONG MOTION SCHEMA DIAGRAM	13

Purpose	13

Event Group	13

Event	13

Bind Group	13

Bind	13

Datum Summary Group	13

SMMessage	13

Datum Summary Group	13

SMMotion	13

COOKED INFRASTRUCTURE SCHEMA DIAGRAM	15

Purpose	15

Core	15

Chan	15

Site	15

SiteT	16

Comp	16

CompT	16

Chan	17

ChanT	17

ChanCTF	17

CookedTF	18

PolesAndZeroes	18

Device	18

External Station Schema Interface	19

Purpose	19

Staion_2_idChan_External	19

Station_External	19

Infrastructure Back End Interface	19

Purpose	19

WAVEFORM SCHEMA DIAGRAM	20

Purpose	20

WaveformDesc	20

Waveform	20

Event	20

Bind	20

Chan	20

META SCHEMA DIAGRAM	21

Purpose	21

Comments	21

EWDBNode	21

Event	21

Bind	21

<EWDB_Table>	22

EWDB_Tablelist	22

Appendix A:  Datatypes	23

# PARAMETRIC SCHEMA DIAGRAM

## Purpose

Depict the relationships between various parametric data entities within the Earthworm DB system.

## Event Group

### Event

Purpose:  	Symbolize a collection of data from all other groups, for some purpose.  An example would be an Earthquake, that contains an origin or two, a magnitude, a mechanism, a collection of picks and some waveform snippets.  This could also be a collection of strong motion data.

#### Parameters

idEvent	Unique record identifier

tiEventType:  	Indicates the type of event: Quake, Blast, Tremor, Strong Motion(?), etc.

iDubiocity:	Indicates whether an event is bogus, or potentially the likelihood that an event is bogus. (Enumerated integer XX)

idComment:	Link to a possible Comment for the Event.

Reviewed:	2000/05/05 DK

## Bind Group

### Bind

Purpose:	Bind records from Core entities to an Event.  Bind is the utility used to associate data with an event.  To associate an origin with an event, you would create a Bind record with the idEvent of the event, tiCore set to identify the Origin table, and idCore set to the idOrigin of the origin record to associate with the event.

#### Parameters

*idBind	Unique record identifier

#idEvent	Indicates the event to which the core record is to be bound.

tiCore 	Indicates the entity type(DB table) to bind to the event

idCore	Indicates the record from tiCore to bind to the event identified by idEvent

Reviewed:	2000/05/05 DK

## Summary Group

### Origin

Purpose:	Contains summary information about an estimate of where a seismic event occurred.  It contains information regarding when an where an event may have occurred.  Error information is included separately in the OriginError table.

Parameters

idOrigin	Unique record identifier

#idSource	Indicates the source of the origin

tOrigin	Beginning time of the event. (seconds since 1970 14.4)

dLat	Latitude of the event (degrees 10.6)

dLon	Longitude of the event (degrees 10.6)

dDepth	Depth of the event below the earth’s surface (in kilometers 7.3)

iGap	Largest azimuthal gap between two stations used in calculating origin data (in degrees 3.0.).

dDmin	Distance from epicenter to nearest station (km 7.2)

dRms	RMS travel time residual.( seconds 8.4)

iAssocRd	Number of readings(introduced for USNSN use) that were associated with this origin.

iAssocPh	Number of phases that were associated with this origin.

iUsedRd	Number of readings that were actually used to calculate parameters for this origin

iUsedPh	Number of phases actually used to calculate parameters for this origin

iFixedDepth	Indicates whether or not a “fixed depth” model was used to calculate the hypocenter. 1=TRUE, 0=FALSE.

{error values}	The next 9 values describe a 3D ellipsoid region within which the estimator is confident that the event originated.

iE0Azm	The Azimuth of the smallest principle error (degrees East of North 3.0)

iE0Dip	The Dip of the smallest principle error (degrees 3.0)

dE0	The magnitude of the smallest principle error (km 6.2)

iE1Azm	The Azimuth of the intermediate principle error (degrees 3.0)

iE1Dip	The Dip of the intermediate principle error (degrees 3.0)

dE1	The magnitude of the intermediate principle error (km 6.2)

iE2Azm	The Azimuth of the largest principle error (degrees 3.0)

iE2Dip	The Dip of the largest principle error (degrees 3.0)

dE2	The magnitude of the largest principle error (km 6.2)

dErLat	The latitudinal error of the origin location (degrees 7.3)

dErLon	The longitudinal error of the origin location (degrees 7.3)

dErz	The vertical error of the origin location  (km 7.3)

tMCI	A time confidence interval (?) (seconds 14.4)

idComment	Link to a possible Comment for the origin.

Reviewed:	2001/05/30 DK

### MechFM

Purpose:	Describe the focal plot of the first ground motions as a result of the event.

*Note: this entity currently contains no focal mechanism related attributes.  It only contains attributes that relate it to other entities.

#### Parameters

idMechFM	Unique record identifier

#idSource	Author of this focal mechanism

#idOrigin	Unique identifier of the Origin used to calculate this is focal mechanism.

idComment	Link to a possible Comment for the mechanism.

Reviewed:	2001/04/11 DK

### Magnitude

Purpose:	Describe the size of a seismic event in terms of some scale, such as duration of shaking, size of largest body wave amplitude, size of largest surface wave amplitude, etc.

Parameters

idMag	Unique record identifier

#idSource	Author of this magnitude.  FK to Source.idSource.

dMagAvg	The summary magnitude value, meaning weighted average of all contributing station magnitudes (between 0.000 and 9.999, no units 4.3)

iNumMags	The number of station magnitudes used to compute the summary magnitude.(9.0)

dMagErr	The possible error of the magnitude (Median Absolute Difference) (Magnitude units 4.3)

#iMagType	Type of Magnitude (MD, ML, MB, MH, etc.)  FK to MagType.iMagType.

#idOrigin	Origin used to calculate this Magnitude.  FK to Origin.idOrigin.

idComment	Link to a possible Comment for the Magnitude.

Reviewed:	2001/04/11 DK

## Link Group

### OriginPick

Purpose:	Describe the relationship between a Pick(phase arrival) and an Origin, and how the measurements of the Pick affected the calculation of Origin parameters.

Parameters

*idOriginPick	Unique record identifier

#idOrigin	Identifier of the Origin record to which the OriginPick is related

#idPick	Identifier of the Pick to which the OriginPick is related

sPhase	The phase that the Origin Author believes that the Pick represents (P, S, etc.)(6 chars)

tPhase	The time that the Origin Author thought the phase actually occurred (calc'd time instead of the observed time that is part of the pick)  (seconds since 1970 14,4)

dWeight	Weight given to the Pick in calculating the Origin location  (5,2) (Interpretation is not described and is possibly Source dependent)

dDist	Distance from station where pick was recorded to epicenter (km 7,2)

dAzm	Azimuth to epicenter from station. (degrees East of North 4,1)

dTakeOff	Emergence angle at source.  (degrees 4,1)

tResPick	The time residual of the Pick (difference between OriginPick.tPhase and Pick.tPhase)(seconds 14,4)

Reviewed:	2000/05/05 DK

### MechFMPick

Purpose:	Describe the relationship between a Pick and a MechFM (focal mechanism), and how the measurements of the pick affected the calculation of MechFM parameters.  (Since MechFM doesn’t do much at this stage, neither does MechFMPick.)

Parameters

idMechFMPick	Unique record identifier

#idMechFM	Identifier of the MechFM to which the MechFMPick is related

#idPick	Identifier of the Pick to which the MechFMPick is related

The combination of (idMechFM and idPick) is unique.

Reviewed:	2000/05/05 DK

### MagLink

Purpose:	Describe the relationship between signal measurements(mostly amplitudes) for a given station and the summary Magnitude that is generated for an event.  A MagLink relationship can occur for all types of Magnitudes/Measurements, but the Magnitude type must match the Measurement type.  For the purposes of this schema, you cannot create a summary ML from Coda Duration Measurements.  MagLink essentially represents a station magnitude.

Parameters

idMagLink	Unique record identifier

#idMag	Identifier of the Magnitude record to which the MagLink is related

idDatum	Identifier of the Measurement(AmpBody, CodaDur, etc.) record to which the MagLink is related.  This is a pseudo-FK to one of the measurement table identifiers (AmpBody.idAmpBody, CodaDur.idCodaDur).

dMag	The station Magnitude calculated based upon the idDatum measurement (3,2)

dWeight	The weight given to the station Magnitude(dMag) in calculating the summary magnitude (Magnitude.dMagAvg), a weighted average of all of the relevant station magnitudes (4,1)

Reviewed:	2000/05/05 DK

### MagType

Purpose:	Describe the type of a magnitude, such as ML, MD, MB, etc.  THIS TABLE SHOULD BE CONSIDERED READ ONLY!!!!!

*NOTE:  This table is not a Link table, it just spatially fit well under the LINK column.

Parameters

iMagType	Magnitude type identifier.  This is not a globaly Unique id type, but a simple integer value that is assigned by Earthworm Central for a given magnitude type.  It is very important that the records in this table correspond to constants in DB API and other code.  So no one should populate this table with their own values.

Identifier of the Magnitude record to which the MagLink is related

sMagAbbrev	This is the code or abbreviation for the magnitude, such as “ML” for Local Magnitude. (4 char)

sMagName	This is the long-hand name of the magnitude, such as “Local Magnitude”. (40 char)

tiMagType	This is the Table Identifier for the Datum elements of the magnitude, such as PeakAmp and CodaDur.

Reviewed:	2001/04/11 DK

## Datum Summary Group

### CodaDur

Purpose:	Describe the relationship between a Pick and a Coda Termination.  (They must be for the same channel).  The meat of the relationship is the time measurements between the Pick occurrence and the termination of the Coda.

Parameters

*idCodaDur	Unique record identifier

#idTCoda	Identifier of the Tcoda (Coda Termination) to which the CodaDur is related. FK to TCoda.idTCoda.

#idPick	Identifier of the Pick to which the CodaDur is related. FK to Pick.idPick.

tCodaDurObs	Time between the Pick time and the Observed Coda Termination time (seconds since 1970 14,4)

tCodaDurXtp	Time between the Pick time and the Calculated Coda Termination time. (seconds since 1970 14,4)

Reviewed:	2001/05/30 DK

### Ray

Purpose:	Provide a better description of multidirectional motion at a station.  A Ray is created by combining data from 3 complimentary sensors at a given Station(Site).  (Ray is not currently implemented as a functional part of this schema.  It has been implemented as a table as depicted by Carl Johnson)

Parameters

idRay	Unique record identifier

tiExternal	External schema table from which this record is derived

xidExternal	RecordID from the external schema table from which this record is derived

tOn	Start time of the Ray measurement (since 1970)

tOff	End time of the Ray measurement (since 1970)

dAzm	Azimuthal measurement of ?  (degrees East of North 4,1)

dSlow	Measure of the slowness of incoming wave(yes slowness is a seismological term.)

(CLEANUP What are the units for slowness?)

idChan1	Identifier of the 1st channel making up the ray.  All channels are from the same site.

idChan2	Identifier of the 2nd channel making up the ray.  All channels are from the same site.

idChan3	Identifier of the 3rd channel making up the ray.  All channels are from the same site.

Reviewed:	2000/05/05 DK

## Datum Group

### Pick

Purpose:	Identify the arrival of a waveform phase at a seismic station(component).

Parameters

idPick	Unique record identifier

#idChan	Identifier of the channel for which this pick was made

#idSource	Identifier of the author of this pick. (Optional)

sPhase	The type of phase that was picked (P,S, etc. )(6 chars)

tPhase	The time the phase arrived (seconds since 1970 14,4)

cMotion	The first motion of the phase (U or D)

cOnset	The onset of the phase {i = impulsive, e = emergent, q = ?}.

dSigma	The quality of the phase in terms of possible error deviation (seconds 14,4).  This

field should probably be tSigma since it is a time value.

Reviewed:	2001/04/11 DK

### TCoda

Purpose:	Identify the termination of a Coda for a given channel

Parameters

idTCoda	Unique record identifier

#idChan	Identifier of the channel for which this TCoda was made

tCodaTermObs	Observed time of the coda termination (seconds since 1970 14,4)

tCodaTermXtp	Calculated time of the coda termination (seconds since 1970 14,4) Calculated by amplitude measurements(CodaAmp) and decay algorithm.

Reviewed:	2001/04/11 DK

### CodaAmp

Purpose:	Identify average amplitude measurements made during a coda of a given channel.  The measurements are used along with a decay algorithm to calculate the termination time of the coda.

Parameters

idCodaAmp	Unique record identifier

#idChan	Identifier of the channel for which this amplitude measurement was made. FK to Chan.idChan.

#idTCoda	Identifier of the coda termination(TCoda) record that this record supports.  More than one amplitude measurements can be used to calculate a coda termination time. FK to TCoda.idTCoda

tOn	Start time of the average amplitude measurement (since 1970 14,4)

tOff	End time of the average amplitude measurement (since 1970 14,4)

dAvgAmp	The average amplitude value between tOn and tOff (microns  9,0)

Reviewed:	2001/04/11 DK

### PeakAmp

Purpose:	Identify the peak amplitude measurements made for a wave phase for a channel.  The amplitude measurement(s) is used to calculate a magnitude for a phase for a channel.

Per Bittenbinder, different types of magnitudes care about different amplitudes taken at different times.  However, almost all magnitudes (Duration and Moment excepted) are derived from at most(for our purposes) two peak amplitudes.  So we store two peak amplitudes and leave it to the individual authors/interpreters of the magnitudes to figure out the significance of the two amplitudes.  (They should be standardized for each type of magnitude)

Parameters

idPeakAmp	Unique record identifier

#idChan	Identifier of the channel for which this amplitude measurement was made. FK of Chan.idChan.

dPeakAmp1	Amplitude #1 of the phase (microns 12,5).

tAmp1      	Time of the amplitude measurement (seconds since 1970 14,4)

tPeriod1	The period between waves at the time of measurement (10,4)

dPeakAmp1	Amplitude #1 of the phase (microns 12,5).

tAmp1      	Time of the amplitude measurement (seconds since 1970 14,4)

tPeriod1	The period between waves at the time of measurement (10,4)

#iMagType	The type of magnitude that this amplitude relates to (ML, MB, MH, etc.)

Reviewed:	2001/04/11 DK

# EXTERNAL SCHEMA DIAGRAM

## Purpose

This diagram depicts the relationships between an Event and its preferred summary information.  It also depicts the relationships between an information Source and an Event, various summary information, and the concept of an External Event that is an event from an external system.  Lastly it depicts the relationship between core EW5 Schema2 tables and External tables, where core may be derived from data in tables external to the EW5 system.

## Event Group

### Event

See Parametric Schema Diagram

## Bind Group

### Prefer

Purpose:	Indicate preferential summary records for an event, such as the preferred origin for an event.

#### Parameters

*idPrefer	Unique record identifier

#idPrefOrigin	Identifier for preferred Origin record for the event

#idPrefMag	Identifier for preferred Magnitude record for the event

#idPrefMech	Identifier for preferred Mechanism record for the event

#idEvent	Identifier for relative event

Reviewed:	2000/05/05 DK

## Summary Group

### Origin

See Parametric Schema Diagram

### MechFM

See Parametric Schema Diagram

### Magnitude

See Parametric Schema Diagram

## Datum Group

### Pick

See Parametric Schema Diagram

### TCoda

See Parametric Schema Diagram

### CodaAmp

See Parametric Schema Diagram

### PeakAmp

See Parametric Schema Diagram

## External Group

### External Table:  DEPRECATED!!!!

Purpose:	A table external to the Earthworm DB core set of tables.  The table is presumably part of or a conversion from another system.  The table may contain data beyond what is stored in the core schema tables and may be referenced by one or more records from one or more core schema tables.

Parameters

xidKey	The table must have a unique key that can be used to uniquely identify a record from the table.  The key is transformed into an alphanumeric string for use as a foreign key by core tables, and stored in the xidExternal field of a core table record.  A unique record must be identifiable by a combination of the tiExternal and xidExternal fields from a core table record (tiExternal identifies the table that the external record is a part of and xidExternal identifies the record within the table.

Parameters	Miscellaneous attribute fields, unknown to the core Earthworm DB schema.

## Others

### Source

Purpose:	Identify the source of an event or summary data.  Source identifies the author of information.  The source is tracked for events as well as individual summary data records for events.

Parameters

idSource	Unique record identifier

*sSource	The text string identifying the source.  This may be anything the source wishes to use and may not necessarily be easily understood by humans (50 chars).  This field must be unique, so two sources cannot have the same sSource.

sHumanReadable	The name of the source, in a format that is intelligible to humans (100 chars).  Example:   the sSource field might contain the ID of the Earthworm module that declared an event, and the sHumanReadable field might contain the name of the seismic network that operates the Earthworm system that declared the event.

idComment:	Link to a possible Comment for the event

Reviewed:	2000/05/05 DK

### ExternalEvent

Purpose:	Associate a DB Event with an event  in an external system, allowing someone to update events that originated from external systems and/or link DB events to events from external systems.

Parameters

*idExternalEvent	Unique record identifier

#idSource	Identifier of the source of the event that is to be linked to a DB Event. FK to Source.idSource.

sSourceEventID	The event identifier given to the event by the external source.  If an event was imported from a USNSN system, and that system gave the event an id, then this is the field that would store that USNSN event id. (40 chars)

idComment:	Link to a possible Comment for the event.

Reviewed:	2000/05/05 DK

# STRONG MOTION SCHEMA DIAGRAM

## Purpose

This diagram depicts strong motion data, and the relationships between it, Events, and Channels.

## Event Group

### Event

See Parametric Schema Diagram

## Bind Group

### Bind

See Parametric Schema Diagram

## Datum Summary Group

### SMMessage

Purpose:	Track information for an Earthworm TYPE_STRONGMOTIONII message.

Parameters

idSMMessage	Unique record identifier

#idChan	Identifier of the Channel from which this message came.  FK to Chan.idChan.

tMotion	Time the author of the message assigns to it.  This could be the trigger time from a bog in the field or the pick time from a data processing (seconds since 1970 14,4)

tLoad	Time the message was first loaded into an EW DB. (seconds since 1970 14,4)

tAlt	Alternate time assigned to the message by some entity. The type of entity that assigned the source is depicted by iAltCode.  (Optional)(seconds since 1970 14,4)

iAltCode	Code specifying the source of tAlt.  (0 – None, 1– Receiving Module,  2 – Database, 3 – Reviewer)

tPGA	Time of the Peak Ground Acceleration for this message.  (Optional) (seconds since 1970 14,4)

tPGV	Time of the Peak Ground Velocity for this message.  (Optional) (seconds since 1970 14,4)

tPGD	Time of the Peak Ground Displacement for this message.  (Optional) (seconds since 1970 14,4)

Reviewed:	2001/04/11 DK

## Datum Summary Group

### SMMotion

Purpose:	Track strong motion measurements for a given channel at a given time.

Parameters

*idSMMotion	Unique record identifier

#idSMMessage	Identifier of the message from which this motion measurement came.  FK to SMMessage.idSMMessage.

iMotionType	Type of motion the measurement is for.  (1 – Acceleration, 2 – Velocity, 3 – Displacement).

dPeriod	The period for a response spectral acceleration(RSA) measurement.  (-1 to indicate a peak(non-RSA) measurement. (seconds 6,3)

#idChan	Identifier of the Channel from which this motion measurement came.  FK to Chan.idChan.  *NOTE:  This is a redundant field.  It is copied directly from SMMessage.idChan, and exists in this table purely for performance reasons, so that SMMotion can be joined directly to Chan without SMMessage.

dMeasurement	The size of the motion.  (cm/s/s for acceleration, cm/s for velocity, cm for displacement).  (10,6)

Reviewed:	2001/04/11 DK

# COOKED INFRASTRUCTURE SCHEMA DIAGRAM

## Purpose

This diagram describes the following things:  the relationships between Site (a physical place in the field where seismic equipment may be placed), Comp (a seismic component sensor), and Chan (a path from a component to digital waveforms or another information source) and their changes over time;  the relationship between a channel and a Poles and Zeros based transfer function that approximates ground motion from a channel’s signal; the relationship between the cooked infrastructure tables and external systems; and the relationship between a channel and the head of a chain of electronic devices.

## Core

### Chan

Purpose:	Identify a unique source of waveform data.  The channel(Chan) identifies a unique path from a component that senses ground motion through the digitizer/digital filter that creates the digital waveforms used by seismic processing systems.  The Chan provides a mechanism for identifying the unique path utilized in converting ground motion into a final set of digital waveforms, so that a set of transfer functions can be calculated to convert the digital waveforms back into ground motion.  Knowing which component in the field recorded the ground motion is not enough to back-out the digital waveforms into ground motion, information is also needed on any signal altering components between a sensor in the field and the last filter/digitizer that generates the waveforms.  Multiple sets of digital waveforms may be generated from one component by transmitting the signal over multiple paths before creating digital waveforms from it(the signal), so it is not enough to track only the sensor in the field, the entire path must be tracked an identified. The Chan table provides the mechanism for uniquely identifying this path.  Much is made of “digital waveforms” in this paragraph, as it represents the most common source of seismic information;  the path starts at ground motion, and ends at the information source where the data is interpreted.  If data is filtered, decimated, or altered in some way once it has become “digital waveforms”, then the Chan does not end at the digitization of the waveforms, but at the point where they are last altered.   One of the key points to a Chan(channel) is how it is defined.  There are two ways to uniquely identify a channel: giving an idChan, or by going to the physical channel and pointing at it.  It is deemed impractical to utilize the pointing method within the database, so the only method allowed in the schema is the use of idChan.  YOU CANNOT USE SCNL(Sta, Comp, Net, Loc) to define a Chan.  You can use a combination of Station Component, Network, and Location to get a list of Chans back, but you cannot use them to define a channel.  This causes a problem in populating the database.  idChan’s are essentially something built within the database, so there is no way for an external system to generate a proper idChan.  Because external systems do not use idChan, this essentially generates a problem: a channel cannot be defined with anything other than an idChan and idChans are not tracked outside of the database.  This is where the first form of identifying a channel becomes useful.  Although external systems don’t use idChan’s as data channel identifiers, there should be people running the systems with a priori knowledge of which incoming data streams correspond to which idChans.  The system administrators should be capable of identifying channels by pointing at them, something that is not possible within the database, and then associating a physical channel with a channel identifier from a system outside the database.  The administrator then creates a lookup table (sometimes referred to as a cheater table) that allows an automated connector to convert an external system’s channel identifier to an idChan.

Parameters

idChan	Unique record identifier

idComment:	Link to a possible Comment for the channel(Chan)

Reviewed:	2000/05/05 DK

### Site

Purpose:	Define a site, the location of zero or more seismic components somewhere in the world, and the site’s fixed, non-electronic attributes.

Parameters

*idSite	Unique record identifier

sSta	The SEED like station code for the site. (7 char)

sNet	The SEED like network code for the site. (9 char)

idComment:	Link to a possible Comment for the site.

Reviewed:	2000/05/05 DK

### SiteT

Purpose:	Define variable(with respect to time) attributes of a site that are fixed for a certain window of time.  SiteT represents the time window and the attribute values for the window.

Parameters

*idSiteT	Unique record identifier

#idSite	Link to a Site for which this record represents a time window

tOff	Ending time of the window (seconds since 1970 14,4)

tOn	Starting time of the window (seconds since 1970 14,4)

dLat	Latitude of the site.  (degrees 10,6)

dLon	Longitude of the site.  (degrees 10,6)

dElev	Elevation of the site above sea level.  (meters 7,2)

idComment:	Link to a possible Comment for the site time window (SiteT)

Reviewed:	2000/05/05 DK

### Comp

Purpose:	Define a seismic component and it’s fixed non-electronic attributes.

Parameters

idComp	Unique record identifier

#idSite	Link to the Site where this component is approximately located. The combination of (idSite, sComp, sLoc) is a unique key.

sComp	The SEED like component code for the component. (9 chars)

sLoc	The SEED like location code for the component. (Can be NULL, 9 chars)

idComment:	Link to a possible Comment for the component

Reviewed:	2000/05/05 DK

### CompT

Purpose:	Define variable(with respect to time) non-electronic attributes of a component that are fixed for a certain time window.  CompT represents the time window and the attribute values for the window.  Station, Network, Component, and Location codes have been added to this table.  They are taken from it’s corresponding Comp and Site tables.  They are fixed attributes of a component, but have been replicated to the CompT table to allow single table look-ups of station location from SCNL.  This has been done purely for performance reasons and creates an opportunity for data anomaly.

Parameters

*idCompT	Unique record identifier

idComp	Link to a Comp for which this record represents a time window

tOff	Ending time of the window. (Seconds since 1970 14,4)

tOn	Starting time of the window. (Seconds since 1970 14,4)

sSta	{DENORMALIZED} The SEED station code for the site to which this CompT is indirectly associated.  See (Site.sSta).

sComp	{DENORMALIZED} The SEED component code for the component to which this CompT is associated. See (Comp.sComp)

sNet	{DENORMALIZED} The SEED network code for the site to which this CompT is indirectly associated. See (Site.sNet)

sLoc	{DENORMALIZED} The SEED location code for the component to which this CompT is associated.  See (Comp.sLoc)

dLat	Latitude of the component.  (degrees 10,6)

dLon	Longitude of the component.  (degrees 10,6)

dElev	Elevation of the component.  (meters 7,2)

dAzm	Azimuth orientation of the component (degrees east of north 4,1)

dDip	Dip orientation of the component (degrees below surface tangent 4,1)

idComment:	Link to a possible Comment for the component.

Reviewed:	2000/05/05 DK

### Chan

Please see the description of Chan at the top of this section.

### ChanT

Purpose:	Define variable(with respect to time) non-electronic attributes of a channel(Chan) for a certain window of time.  The attributes, that are variable for the channel over time, are fixed for a certain time window.  ChanT represents the time window and the attribute values for the window.

Parameters

*idChanT	Unique record identifier

#idChan	Link to a Chan for which this record represents a time window

tOff	Ending time of the window. (Seconds since 1970 14,4)

tOn	Starting time of the window. (Seconds since 1970 14,4)

#idCompT	Link to a CompT record that defines the attributes of the component that is the source of the channel for atleast the (tOn  - tOff ) time range.

#(#idDeviceSlot	Link to a DeviceSlot record that represents the last device slot in the channel.  DeviceSlot is part of the “Raw Infrastructure” schema that describes channel details and instrument response.

iPlexor)	Combined with idDeviceSlot to link to a particular output pin on the last Device Slot in the channel.

idComment:	Link to a possible Comment for the channel time window(ChanT)

Reviewed:	2001/04/11 DK

### ChanCTF

Purpose:	Depict the relationship between a channel time window (ChanT) and a Transform Function, and the attributes of that relationship.

Parameters

*idChanCTF	Unique record identifier

#idChanT	Link to the ChanT record for which there is a transform function. FK to ChanT.idChanT.  This is the primary key because there is at most 1 transform function per channel time interval(ChanT), therefore ChanCTF is just an optional set of attributes for a ChanT.

#idCTF	Link to the transform function to be used for the ChanT.  FK to CookedTF.idCTF.

dSampRate	Sample rate of data for the channel during the ChanT time window. (Samples/Second 8,4)

dGain	Gain for the channel during the ChanT time window. (multiplicative scalar of the signal amplitude, floating point decimal)

Reviewed:	2000/05/05 DK

### CookedTF

Purpose:	Depict a Poles and Zeroes based transform function for approximating ground motion from a set of digital waveforms.  A single transform function may be associated with multiple time windows from multiple data channels.

Parameters

idCTF	Unique record identifier

sFuncDesc	A name or brief description of the function (50 chars)

Reviewed:	2000/05/05 DK

### PolesAndZeroes

Purpose:	Depict a pole or zero member of a transform function as a complex number.

Parameters

#idCTF	Link to the cooked transform function(CookedTF) to which this record belongs. FK to CookedTF.idCTF.

idPZ	Unique record identifier.

cPZType	A flag indicating whether the record represents a pole or zero. (1=Pole, 0=Zero)

dReal	The real portion of the complex number. (14,7)

dImaginary	The imaginary portion of the complex number. (14,7)

Reviewed:	2000/05/05 DK

### Device

Purpose:	Represent a device in a waveform data channel.  The attributes of the table are not depicted here.  The Device table is part of the Infrastructure (non-cooked infra.) schema, and is only described here as the link from the Infrastructure Front End(cooked) to the Infrastructure Back End(Raw or just straight Infrastructure).  Please see the Infrastructure Schema diagram for a description of Device.  *Note:  As of this writing, the Infrastructure Schema has not been released, so you will not find an Infrastructure Schema diagram.

Parameters

idDevice	Unique record identifier

Reviewed:	2000/05/05 DK

## External Station Schema Interface

### Purpose

Describe how external systems that use some derivative of SCN, interface with the Earthworm DB Cooked Infrastructure.   This interface is not part of the main EW DB Schema, but serves as an interface between external systems and the Earthworm DB system.

### Staion_2_idChan_External

Purpose:	Serve as a lookup(read cheater)  table to convert from a StationID to an idChan

Parameters

*idChan	Unique record identifier.  Link to a Earthworm DB Chan

StationID	Primary Key link to an External Station.

Reviewed:	2000/05/05 DK

### Station_External

Purpose:	External station table.  Part of an external system (that uses SCN*) that depicts information about the source of a data stream.

Parameters

StationID	Locally generated sequence number  (not generated as part of the EW DB schema)

Sta	SEED like Station Code (10 char)  The combination of (Sta, Chan, Net, and Loc)is a unique key.

Chan	SEED like Channel Code (10 char)

Net	SEED like Network Code (10 char)

Loc	SEED like Location Code (Can be NULL, 10 char)

Lat	Latitude of the component for this data stream.  (degrees 10,6)

Lon	Longitude of the component for this data stream. (degrees 10,6)

Elev	Elevation of the component for this data stream.  (meters 12,4)

Description	Brief description or comment related to the data stream. (char 100)

Reviewed:	2000/05/05 DK

## Infrastructure Back End Interface

### Purpose

Describe how the Front End of the infrastructure schema (known as the cooked infrastructure) interfaces to the the Back End of the infrastructure schema (known as the raw infrastructure).

The ChanT table represents as channel(Chan) for a given time interval defined by tOn-tOff.  For the Channel Time Interval(ChanT), there exists at most a single link to the infrastructure back end.  This link connects the ChanT record to an output pin on the last device in the channel for the duration of the time interval.  So ChanT is linked to ModuleTemplate, which identifies an output pin on a device slot.  As a side effect of the key structure for ModuleTemplate, ChanT is also linked to DeviceSlot, since idDeviceSlot is a component of the primary key for ModuleTemplate.  Please see the Raw Infrastructure Schema documentation for more information. (Probably more information than you want.)  The Raw Infrastructure Schema documentation currently resides in a separate document (ewdb_infra_schema_description_v2.15.doc or something with a slightly different version number.)

# WAVEFORM SCHEMA DIAGRAM

## Purpose

This diagram describes the method for storing waveforms along with the method for associating waveforms with an event.

### WaveformDesc

Purpose:	Describe a waveform snippet and the method used to store that waveform in conjunction with the DB.

Parameters

idWaveform	Unique record identifier.

#idChan	Link to the channel(Chan) from which the waveforms came.

tStart	Starting time of the waveform snippet. (Seconds since 1970 14,4)

tEnd	Ending time of the waveform snippet. (Seconds since 1970 14,4)

iDataFormat	Data format of the waveform snippet. Enumerated value. (1=Earthworm Binary TraceBuf) (9,0)

iByteLen	Length of the snippet in bytes. (10,0)

iStorageType	The method used to store the waveform snippet in conjunction with the DB. (1=Waveform Table) (9,0)

sStorageInfo	String providing additional information about the storage of the waveform snippet.  This could be a filename or anything else.  It’s meaning is dependent upon the value of iStorageType. (40 chars)

Reviewed:	2000/05/05 DK

### Waveform

Purpose:	Store a binary waveform snippet within the DB.

Parameters

#idWaveform	Unique waveform record identifier.  This is a FK to WaveformDesc.idWaveform.  A Waveform record is one of the ways that a waveform snippet can be stored in conjunction with the DB.  Each waveform record must have a WaveformDesc that describes it and how it is stored.

binSnippet	The actual binary waveform snippet.  (long raw 4GB max)

Reviewed:	2000/05/05 DK

### Event

See Parametric Schema Diagram

### Bind

See Parametric Schema Diagram

### Chan

See Cooked Infrastructure  Schema Diagram

# META SCHEMA DIAGRAM

## Purpose

Depict generic implementation details for the schema, such as how a key is obtained, how table identifiers are handled, how comments are handled, and how multiple DB nodes are handled for use in a distributed setup.

### Comments

Purpose:	Provide a generic comment functionality.  Any core table that could make use of a comment utilizes the Comments table for the comment in lieu of embedding the comment in the table’s data record.  By abstracting the comment from the data record, it allows for much denser record storage while still providing  the ability to associate long detailed comments with a particular data record.

Parameters

idComment	Unique record identifier

sComment	A text field containing the comment.  (4000 characters).

Reviewed:	2000/05/05 DK

### EWDBNode

Purpose:	Provide a means for identifying a unique database node across a system of distributed databases.  The EWDBNode number is used in transforming locally generated sequence numbers into keys that are unique across multiple database nodes.  Each node has a unique node number, that when combined with a sequence number that is locally unique, can generate a key that is globally unique.

Parameters

EWDBNodeID	Unique record identifier (4 digits)

EWDBNodeName

Arbitrary name of a database node (40 chars)

idComment:	Link to a possible Comment for the node.

iIsMyNodeID	Flag indicating that the current record describes the local node.  Records representing all of the database nodes may be stored in the local node table, but only one record may be flagged as the local node.  This is used to figure out which EWDBNodeID to use in creating globally unique records in the local database node. (1=TRUE, 0=FALSE).

Reviewed:	2000/05/05 DK

### Event

See Parametric Schema Diagram

### Bind

See Parametric Schema Diagram

### <EWDB_Table>

Purpose:	This is a template for the schema core data tables.  It depicts that each table is required to have a numeric identifier that is globally unique for that table variety.

Parameters

<idKey>	A unique record identifier.  The field name is not necessarily “idKey” but is of the “id” variety.  It is a combination of a locally unique sequence number, and a EWDBNodeID, so that a key is generated that is globally unique to the specified type of table.  Different sequence generators may be used for different table types, such as Origin and Event, but an <idKey> field in a local Origin table is guaranteed to be unique for all Origin tables across all database nodes.  Other nodes may have a record with the same key, but that id describes the exact same record in all nodes.

Reviewed:	2001/04/11 DK

### EWDB_Tablelist

Purpose:	Provide a means of identifying a unique table.  Currently the tablelist only contains a list of local tables with no EWDBNodeID specified.  This should probably be changed so that an EWDB_Tablelist record uniquely identifies a table across all database nodes. EWDB_Tablelist is used to resolve table identifiers(tiXXX) stored in several of the core tables.  The schema uses the Bind table as a generic associator to associate records from various tables with an Event.  Each Bind record contains a tiCore field that identifies the table whose record is to be bound to the event.  The table is specified with a Table Identifier which is envisioned as a local sequence number.  The number is then used to look up the name of the data table in the EWDB_Tablelist table. EWDB_Tablelist acts as a lookup table to go from tablename to table identifier and vice versa.

Parameters

*idTable	Unique record identifier

sTableName	Primary Key:  The name of the table.

Reviewed:	2001/04/11 DK

# Appendix A:  Datatypes

The following datatypes are utilized in the diagram

i:	Integer  (presumably 4 byte, but probably within the range of –999,999,999 to 999,999,999)

t:	Time   in double form, represented in seconds.  For absolute times, it is seconds since 1/1/1970 (Julian)  Of the form %10.4f  or (Number(14,4) in Oracle)(1987654321.1234 or 9999999999.9999)

d:	Double  within the limitations of a standard C double.  Because Oracle does not define numbers in terms of floating points, the precision is more precisely defined in the actual tables.

ti:	Table Identifier.  This is a variable that can uniquely identify a table.  Theoretically, it should be capable of identifying a table across all EW DB installations, but currently (as implemented) it can only identify a table within a single database.  Conceptually a table identifier is not a number nor a string, but something that is able to identify a table.  In the EW DB implementation, table identifiers are implemented as a number that is used to look up a table name in the EWDB_Tablelist table.  This

should later be changed to be a combination of a EWDBNodeID and a table code, similar to an ID type.

id:	A Core table ID.  This is a 13-digit integer of the following form XXXXYYYYYYYYY, where the X’s represent the 4-digit schema2 installation ID that is responsible for the record, and the Y’s identify the 9-digit unique ID value for the table at that installation.  Example:  you run across and Origin record with idOrigin = 12000000123, you can glean(if that’s a word) that it came from DB installation 12, with and id of 123 generated for that installation’s Origin table.  The 123 is unique within the origin table, and the 12 is unique among the installations.  There could also be a record in the Event table at your installation, with idEvent = 12000000123.  The two records are unrelated, because the 123 for one record was generated to uniquely identify it among other origin records, and the other 123 was generated to uniquely identify it among other event records.  However, if you go to two separate installations, and find

at each of them an origin record with idOrigin=12000000123, then those records are the same origin.

xid:	A recordID for an external table.  As implemented this is an ASCII string representation of a key for an external table no more than 16 characters long.  For a Core Origin record generated from an Earthworm Schema Origin record, this would be the OriginID from the Earthworm Schema Origin record.

c:	A single ASCII character.  ‘b’ for example.

s:	An ASCII string. “The lazy brown programmer jumped over the banana peel,” for example.